MEMORANDUM

Date:		October 6, 2009

To: 		Rodney Thomas, CSSI

From: 		Richard Bartel, CSSI

Subject:	ADS-B / C-ARTS SRMD Review

Per your email of September 22, 2009, I have surveyed the following documents provided, in light of  the SRMD processes, and am attaching my comments, within my technical competence and experience, without duplicating (but considering) comments already received by the FAA (as supplied from previous versions).  I understand that this has been a short time for review, and that you will be consolidating these Comments with others from within CSSI, and receiving further documentation in the near future.   Please attach this Memorandum to the transmittal to FAA and the compiled Comments.

(1) ADS-B/CARTS Critical Services-Terminal ATC (64 pages)

(2) ADS-B/CARTS Subsystem Hazard Analysis (386 pages)

(3) ADS-B/CARTS Performance Evaluation (183 pages).

SRMD Review and Comments:

1)  ADS-B/CARTS Critical Services-Terminal ATC:

Chart p. iv : 	In the Chart, Table 1-1, it might be helpful to know which hazards were associated with an Initial High Risk item, if applicable.

pp. iv:  	A statement is made that SSHA / SHA addresses local procedures and maintenance and certification, and that a separate O&SHA will be developed for national issues in September, 2010.  Does this refer to terminal use?

Par 3.2.3 (p. 2):  In this SRMD, there is reliance on the vendor (ITT) to convene a “system safety working group” to provide input to this SRMD without information on how the FAA procedurally utilized the vendor’s group input (i.e. did the FAA Panel apply the same processes to the vendor input as it would to input from other Panel membership or “breakout” groups?).  The vendor also gave periodic briefings.

Par. 4.1 (3) (p.2)  	Assumptions indicate that “GPS spoofing” is a security risk, and that it assumes that a SBS Program Security Certification and Authorization Package (SCAP) will address security issues (implies a future event).

It is not clear if the term “GPS Spoofing” includes a tampering with the signal(s) received by the aircraft, or also includes an intentional introduction of false position, ID, etc. data on board an aircraft, or elsewhere, whether or not with malicious intent (“spoofed GPS data”).  As indicated in these comments,  there might also arise attempts to shroud / cloak the presence of one aircraft, or “spoof” its actual trajectory, so as to hide its track or point of arrival, resulting in traditional security issues (i.e. terrorist threats, etc.), becoming a subset of possible surveillance avoidance or manipulation.

In any case, regardless of intent, such situations would present a safety hazard to other aircraft.  Similar behavior has been noted in current environments, such as flying low below coverage (i.e. in the future, below 1,500 to 3,000 ft in the Gulf of Mexico, even with ADS-B virtual radar), turning off transponders, and flying in controlled airspace in IMC conditions without a flight plan or clearance (permitted in Class G airspace) or at VFR altitudes with transponder off while mixed in with IFR traffic (presenting particular hazards during climbs and descents of any other nearby aircraft).

System design, and/or Requirements, may need attention, as this hazard with known unresolved mitigations (see semi-annual high risk hazard report), and the issues discussed below (also see: General Discussion), may indicate the presence of a systemic system design hazard which might be resistant to traditional mitigations within programs.

Par. 4.3 Assumptions (p. 3):  There may be other sources of position information beyond GPS which could be considered as backup(s) or resononableness tests for fault logic.  The consideration of these additional inputs to reasonableness testing should be mentioned in applicable mitigation strategies as guidance to future SRMD revisions, and there should be elaboration on the assumptions made in RTCA DO-242A (par. 3.3.6.5) (See: Appendix A, Subsystem Hazard Analysis, Table Hazard A1, Existing Controls and Requirements, p. 8) which is conditional on the ADS-B system being given “correct source data.”).    Within the SRMD, it appears that the controller will be relied upon to detect “suspected fusion anomalies” (See 6.3.2, line 1073), without commentary on any automation features to alert for such “anomalies”, and suggesting a definition, reporting parameters, tracking, and training for “anomalies” noted or encountered.

General Comments:

An FAA semi-annual report on High Risk Hazards dated September 30, 2009 , indicate that there are a number of ATO related High Risk Hazards which have a “Not implemented” status for certain mitigations.  Some of these also involve SRMDs or Programs with similar integration of ADS-B into existing and future systems, some of which indicate systemic issues with regard to data validation / verification / fusion - and aircraft identification and system and controller verification.

While reviewing documents and in conversations relating to ADS-B sensor integration into existing systems, there appears to be an overall ambiguity across ADS-B related programs, regarding whether or not, or to what extent, data fusion is accomplished from end to end, i.e. are all of the attributes of data received from the ADS-B equipped aircraft and from ground systems, when processed by FAA-external vendor(s) (i.e. ITT), and then when processed within FAA, utilized and displayed in a manner which meets general ATC and security system expectations, and goals, and ultimately, overall surveillance system requirements across Programs ?

I believe that this is generally unclear, since some programs plan to integrate ADS-B in differing ways.  For example, technical or equipment constraints, at least initially, may require that some ADS-B data go through multiple conversions and “dumbing down” before being displayed to controllers or other decision makers on existing equipment in the Gulf of Mexico, and possibly other projects.

There has been some indication that most, if not all, ATC display target blocks will indicate whether or not an aircraft has flight planned itself as ADS-B equipped or capable, but there may be insufficient or no verification that the target is actually resulting from verified, validated, and/or resolved ADS-B sensor data, and the “target” is also verified with other sensor data (and flow information) which is not inconsistent with the displayed target or track, and represents the same aircraft.  The discussions of “data fusion” should have executive summary definition and elaboration, particularly in light of any reliance on controllers to detect “anomalies” between surveillance modes.

Some system documentation indicates that reasonableness tests are implemented in software to discard data as “bad” for suppressing its display to controllers, but there does not appear to be a specific unbounded hazard analysis for situations in which a target would be injected into ADS-B data streams where no aircraft actually exists, or where the aircraft reports a position (or flight path / trajectory) other than the one on which it is actually flying (even though the aircraft is flight planned with the same or similar intent or initial flow), shrouding or cloaking its present flight behavior, or reports its position to correspond with another aircraft’s radar or transponder sensed track.  Current analysis in this area seems to focus on “hacking” with respect to information technology issues (IT), and should also separately focus on spoofing and surveillance avoidance as a safety hazard with a security related subset of behaviors.

In a security context, a pair of aircraft, one being tracked and controlled, and the other not, could act in concert to conceal the intent of one of the two aircraft (it is unknown if this has been addressed in the Security context (i.e. SCAP for SBS and other ADS-B implementing Program Office(s))).

Not all false data scenarios could occur within the context of a security threat, but could occur in situations where the aircraft operator simply desires to remain anonymous or execute trajectories which they otherwise would not be able to negotiate or obtain.

In any of the cases above, such behavior would present a safety hazard to other aircraft, notwithstanding intent.  One demarcation between a safety and a security issue is motivation or malice, and it is intuitive that security related events will have a small likelihood, but higher generalized consequence or overall system “severity” (with safety as a component).

In the context of future NextGen capabilities, as systems evolve to allow “shared” or “self” separation, these behaviors may emerge in greater numbers, and early mitigation in system design and in policies may be desirable.

One source of hazard might relate to communications between the ADS-B “provider” (ITT) as an FAA contractor, and the data integrity tracking and monitoring within the vendor’s network (i.e. between the aircraft and the FAA).  Other issues which should be addressed are those resulting from differing implementations by avionics manufacturers (this issue has also surfaced with TCAS (i.e. at RTCA SC 218)).

In general, the ADS-B contractual relationship might limit the extent to which the ITT vendor’s involvement could integrate further into FAA’s air traffic control function as a matter of U.S. policy or law , and may provide challenges to FAA in safety assurance in the future.

October 6, 2009

Richard C. Bartel

Safety Engineer

CSSI, Inc.

8201 Corporate Drive, Suite 750

Landover, MD 20785

(202) 863-3668

rbartel@cssiinc.com

(410) 903-2759

rcbartel@yahoo.com

It appears that the FAA schedule for ADS-B/CARTS is as follows:  “The Final ADS-B/CARTS SRMD will be submitted on October 19. It is to be forwarded to AOV for approval of the mitigations for any Initial High Risk hazards and mitigations allocated to organizations external to the FAA ATO line of business. Relevant external organizations will sign their approval of the mitigations prior to sending them to AOV for approval as per Order 1100.161. Final approval is sought by October 30, 2009 to support declaration of IOC. It will include and updated Executive Summary, Final SSHA/SHA, Final SSAR including V&V status of safety requirements, any open PTRs or DRs, Final SBSS & CARTS Operational Test Quick-Look reports, and a DO-278 Gap Analysis Report for the ITT SBSS.”  (SRMDADS-B/CARTS Critical Services-Terminal ATC, p. ii). Verification testing is planned at Louisville (SDF), and IOC is scheduled for October 30, 2009.  An updated SRMD, with added verification and test results, is expected October 20, 2009, addressing RTCA DO-278 issues as well (we should look at RTCA DO-178 in comparison), and a Briefing is expected for October 23, 2009.

See: White Paper “GPS Backup For Position, Navigation and Timing Transition Strategy for Navigation and Surveillance”, August 22, 2006, By Dr. Robert Lilley , Gary Church, Michael Harrison (Aviation Management Associates, Inc., 1101 King Street, Suite 325, Alexandria, VA 22314; 703-518-9923), for FAA, ATO-W Navigation Services Cooperative Agreement 06-G-001; Also see: JPDO Paper No. 08-0003 (7/15/2008), “FAA’s Surveillance/Positioning Backup Strategy Alternatives Analysis”.

Where RTCA documents are referenced, they have been sometimes used to formulate “Requirements” and Controls, which is generally reserved to Accredited Standards Organizations (i.e. ANSI certified) when “Standards” are being formulated by Government for incorporation into regulatory documents (i.e. ADS-B proposed Regulations, etc.), and possibly discretionarily in FAA Advisory Circulars with incorporated references, or contractual provisions with vendors for procurement.  The posture of documents created by Federal Advisory Committees (i.e. RTCA DOs) is unclear, particularly if Advisory Committee work in certain situations could be construed as “negotiated rulemaking.”  (See 5 USC 561 et seq.). (See references to proposed ADS-B regulations as basis for pilot and/or controller expected actions / functions, underlying some parts of the risk assessments in this SRMD; the concern might arise if the RTCA proceedings / documents generated the regulatory proposal(s), which in turn become part of a baseline for SRMD risk assessments).

Pursuant to Order 1100.161, Air Traffic Safety Oversight, Sec. 3.2.a:  Presented as prepared by Michael Virga (AJS-23), from Mark DeNicuolo (AJS-23) on October 1, 2009 at the FAA ATO SRM Integration meeting.

Aircraft pairs flying in tandem at close proximity, for example, in order to appear as one target, has already been noted in drug interdiction situations.  In an ADS-B sensor situation, the aircraft would not need to fly in pairs, but would only have to have access to the surveillance data of the other aircraft (i.e. possibly from “other authorized user systems” (p. 1)), or be in an ADS-B sensor only environment (i.e. “virtual radar”, such as parts of the Gulf of Mexico)).  These issues would seem to have higher likelihoods in mixed traffic situations, and would vary by traffic density (i.e. terminal environments).

See:  “Simulated Safety Risk of an Uncoordinated Airborne Self Separation Concept of Operation”, Henk Blom, Bart Obbink, G.J. Bakker, Air Traffic Control Quarterly, Vol. 17, No. 1, pp. 63-93 (2009); also see: Metrics for Traffic Complexity Management in Self-Separation Operations”, Husni Idris, Robert Vivona, David Wing (supra, pp. 95-124).

Mitigations by other JPDO Agencies have included encryption of GPS signals.  Tampering with and spoofing GPS signals themselves has been looked at by FAA and other organizations.  Also, as discussed herein, airborne GPS position data received by navigation and surveillance systems might be able to be “Spoofed” , presenting possible security and/or conflict or collision risks (see: Argonne National Laboratories, LAUR-03-2384, The Journal of Security Administration 25, 19-28 (2002),  “A Simple Demonstration that the Global Positioning System (GPS) is Vulnerable to Spoofing” Jon S. Warner, Ph.D. and Roger G. Johnston, Ph.D., CPP, Los Alamos National Laboratory, Los Alamos, New Mexico, 87545.  Various commentators might influence public perceptions (i.e.  “ADS-B … Terrorist’s Dream, Security’s Nightmare” at http://www.airsport-corp.com/adsb2.htm ).

For example, the status of reliance upon ITT within FAA’s air traffic control functions, as an Inherent Government Function is unclear (c.f.: OMB Circular A-76 (http://www.whitehouse.gov/omb/circulars/a076/a76_incl_tech_correction.pdf).  Such jurisdiction might be plenary, original, and/or exclusive.  Executive Orders have treated ATC both ways (EO 13180 (65 FR 77493 (12/11/2000) (inherent), vs. EO 13264, 67 FR 110 (6/2/2002) (not inherent)).